Appendix II

Functional Flows

Matlab Code

User’s Manual

Version 1, June 2007.

Written by

Stephen Murray,

Marisa Escobar, and

Greg Pasternack.

Developed in Matlab 7.

Hydrologic Sciences Graduate Group

Land, Air, and Water Resources Department

Hydrologic Sciences Graduate Group

Land, Air, and Water Resources Department

University of California, Davis

ACKNOWLEDGEMENTS

The Functional Flows model is presented in the PhD dissertation project authored by Marisa Escobar, and advisor Greg Pasternack. The research and methods for the development of the Functional Flows model are documented in

Escobar, M.I. 2007. Evaluating Streamflow to Characterize Ecological Functions of Stream Physical Habitat. University of California, Davis.

The Matlab code presented in this manual was programmed by Stephen Murray.

Support for the dissertation project and the Matlab code development was provided by the John Muir Institute for the Environment at UC Davis, Office of Graduate Studies at UC Davis, and the Hydrologic Sciences Graduate Group at UC Davis.

Table of Contents

Introduction	4

General Summary	4

System Requirements	6

Installation	7

Running the program	7

Entering the units system	7

Entering the water surface slope	7

Entering hydraulic geometry power regression data	8

Entering mean grain size (D50) data	10

Entering daily flow data	11

Entering extra shear stress terms	13

The non-uniform change in height term, dh/dx	13

The non-uniform change in velocity term, U/g*dU/dx	14

The non-steady term, 1/g*dU/dt	16

Entering the Functionality Table	16

Visualizing the Inputs	20

Changing Inputs	21

Interpreting the outputs	22

The first set of figures	22

The second set of figures	23

The third figure type	24

The fourth set of figures	25

The fifth set of figures	26

The sixth figure type	27

The seventh set of figures	28

The Output File	30

Sensitivity Study	31

Tips and Troubleshooting	32

Matlab Coding Acknowledgements	33

# Introduction

## General Summary

The main purpose of this program is to determine the functionality for in-stream and riparian species of a user specified river cross section and water year.  The user must have a good understanding of the concepts and goals of the program for it to produce useful data. The user must have a target species, must understand their lifecycle, and must know what specific stages of the lifecycle are of interest.

The user will be required to define ecological functions, which are the ways in which organisms interact with their physical habitat. For instance, ecological functions for salmon are spawning, embryo incubation, fry emergence, etc. Also, the user will be required to input information about the times of the water year in which they occur and overlap. The water year begins on October 1st and lasts until the 30th of September.

Functional flows are based on two criteria.  The first is the estimation of the shear stress between the flowing water and the riverbed.  The second is the time of year during which each ecological function occurs and the range of shear stresses that are functional for each ecological function. If the shear does not fall in a functional range or if the given day does not fall in the duration of an ecological function, the river cross section will not be functional for the specified ecological functions based on that particular day’s flow data.

The Functional Flows program allows for the selection of standard shear stress values for predetermined ecological functions (i.e. for salmon spawning, embryo incubation, embryo emergence), or user specified values.  The program and results are highly dependent on shear stress ranges and so it is recommended that the user defines sensible and defendable values.

For the program to determine if a certain riverbed cross section is functional or not for an ecological function, the program needs the necessary inputs to estimate shear stress from the governing equation:

τ0=ρRρs−ρD50S−∂h∂x−Ug∂U∂x−1g∂U∂t		(1),

where:

τ0 is the nondimensional shear stress,

ρ is the density of the water,

R is the hydraulic radius,

ρs is the density of the riverbed sediment,

D50 is the mean grain size of the riverbed gravel,

S is the water surface slope in units of length/length, and

the differential terms are non uniform/non steady terms (explained later).

The nondimensional shear stress is graphed versus the nondimensional flowrate shown in equation 2:

Q=QD502gD50	(2),

where:

Q* is the nondimensional flowrate,

Q is the flowrate, and

g is the acceleration of gravity.

From equations 1 and 2, the program shows the user the values of Q* which produce functional values of τ0.  Because the model used in Functional Flows nondimensionalizes these terms, the user must be prepared to input additional details about the river and riffle itself.

The hydraulic radius changes daily because it is directly proportional to the daily flowrate in the river.  The Functional Flows program uses the Manning’s Equation to compute the power regression between stream flow and hydraulic radius, as shown in equation 2:

R=cQf	(2),

where:

R is the hydraulic radius,

c is the power function coefficient,

Q is the daily water flowrate calculated from the Manning’s Equation, and

f is the power function power.

The geometry of the cross section is used to calculate incremental values of hydraulic radius and Manning’s Equation is used to calculate their corresponding discharge values.  A power regression is used to obtain c and f to parameterize the cross section.  With c and f, it is possible to obtain R for any Q.

The user must input values of Q, the flowrate in the river each day, in order to compute the daily shear stress according to Eq. 1.

The program takes the user inputs to find a shear stress at the riverbed for each day within a water year.  Then, it compares these shear stresses to the functional shear stress ranges provided or selected by the user.  If the daily shear stress falls within a functional range for a certain ecological function, the day is said to be Geomorphically Functional, that is, the shear stress lies at a functional value.  In addition, if the given day lies within the timing of a given ecological function, the day is said to be Ecologically Functional.

The advantage of this program is that the user can create as many ecological functions and ranges for shear stress as desired.  The time of year, duration, number of ecological functions, and values of functional shear stresses can easily be varied to investigate different behaviors and species.

The remainder of the User’s Manual will focus mainly on proper data entry in order for the production of valid results.

## System Requirements

To run the Functional Flows program, users are required to have Matlab installed.  Although the Functional Flows program was mainly designed in Matlab 7, older versions of Matlab (such as 6.5) should still support the program.  The program does not require many complicated built-in functions, and therefore, older versions of Matlab without the newest functions should still be able to get clear, organized results.

The only known difference between an older version such as 6.5 and the newer version 7 occurs in some of the basic legend formatting.  In older versions of Matlab, the legend may contain extra entries such as “Location”, or “SouthEast” in an effort to interpret the newer legend formatting in Matlab version 7.  This in no way affects any of the data or results for any analysis.

Versions older than 6.5 have not been tested and it will be left to the user to decided if he or she desires to use Functional Flows on an older version.

If the user has Matlab installed, the time for computations should be negligible for most inputs and analyses.  The output file is a simple text file that should rarely take up more that 500 KB of disc space.

## Installation

All of the necessary files for the Functional Flows program are included in the downloadable zip file.  Extract these files to any folder on the user’s computer.

# Running the program

To run the program, open Matlab and change the current or working directory to the folder in which the Functional Flows files lie.  From the command prompt, type FF and the Functional Flows program will execute.  Please note that any previously stored information in Matlab will be erased, and any opened figure windows in Matlab will be closed.

## Entering the units system

The program firsts asks for the unit system in which to work.  Select 1 for the Metric system or 2 for the English system under the Units System menu.  This is an absolutely critical step because every calculation the program performs uses constants and inputs in the unit system the user selects.  Therefore, check to make sure all of your input data and files are formatted using the correct units before proceeding further with the program.

## Entering the water surface slope

The next step is to enter the water surface slope of the riffle, or river cross section, to be analyzed.  The water surface slope is a measure of the difference between upstream and downstream elevation change divided by the distance, a simple dy/dx measurement at the water surface.  This quantity is dimensionless, and may be entered as one constant value, or as two values:  a high stage water surface slope, and a low stage water surface slope.  If you would like the program to use interpolated values between a high and low slope, select choice 2 under the Water Surface Slope menu.  Both the power regression calculation and the shear stress calculation use these values spanning the depth of the river.

If the only data available for the cross section is one water surface slope value, choose option 1 under the Water Surface Slope menu and the program will use a constant value for the calculations.

## Entering hydraulic geometry power regression data

As mentioned in the general summary in the Introduction, the program computes the hydraulic radius from the geometry of the cross section and the water flowrate from the Manning’s Equation.  The values of c and f obtained from the power regression and the daily flowrate for a given water year (input later) are used to calculate R values for any flowrate value.

If you already know the values of c and f for your riffle, choose option 1 under the Hydraulic Geometry Power Regression Data menu and manually enter these values.  Be very careful that the method of calculation by which you obtained these values uses the same unit system as those you selected earlier.  Testing by the programmers found that values of c and f do not transfer from one system of units to another.

If you do not already have values for these Manning’s variables, the program will calculate them after you have entered the cross sectional geometry of the riffle.  To do so, you must first ensure you have a suitable input file.  The program will only recognize an input file with two (2) columns of data.  The first column of data is the X, or horizontal measurement of the cross section.  The second column is the Y, or vertical measurement of the riffle’s cross section.  Figure 1 is a quick reference to reiterate:

Column 1		Column 2

X Data			Y Data

Figure 1:  Summary of cross sectional geometry file format.

The two columns may have spaces, tabs, or a combination of the two between them, but they may not have any titles or footnotes or anything of the sort.  The columns need to start and end with data, nothing else.  Figure 2 shows an example of the beginning of an appropriate file:

78.012621		93.119278

78.554537		93.167932

80.828093		93.372058

83.632481		93.466373

87.771881		93.523718

97.011834		91.453356

100.186215		91.025702

127.927425		87.867644

133.725182		88.296913

143.011676		88.621746

150.264198		88.889977

151.364383		88.881037

Figure 2: An example of a properly formatted cross sectional geometry file.

Once you have a properly formatted input file, copy it into the folder in which the other Functional Flows files reside.  Next, select option 2 from the Hydraulic Geometry Power Regression Data menu and enter the name of this file INCLUDING the appropriate file extension.  Therefore, if the input file shown above was a text file named Test_Riffle, the proper input would be Test_Riffle.txt after selecting option 2 from the Manning’s Data menu.  An error will occur if you forget to include this, because the program searches for the name and file type you entered to load into the analysis.  Also, if you forget to copy the file into the Functional Flows folder, an error will occur because the program cannot find the file in the current directory.

The next step is the power regression stage increment.  The stage increment tells the program to analyze the cross section every time the vertical measurement (Y) reaches a multiple of the stage increment.  For smaller the stage increments, the power regression calculation becomes more accurate, yet slower.  Depending on the cross section, a reasonable value of the depth increment is half a foot, or about 0.17 meters.  These values should give a good representation of the cross section without taking minutes to compute.

After the stage increment is set, the value of the Manning’s n must be entered.  This is usually a small number which can vary greatly depending on riverbeds.  The user is encouraged to research this and simply enter an appropriate value.  For instance, a standard value of 0.043 is frequently used in gravel bed rivers.  The popular program WinXSPro uses smaller values such as 0.015 – 0.017 as default values.  The manual of WinXSPro contains additional information about the Manning’s n.

Finally, the program displays the cross sectional geometry and asks you where you would like to stop the stage calculations.  This choice is included because many times the measured starting and ending elevations (Y coordinates) of the river do not match up exactly.  If you would like to tell the program where to stop the at-a-stage calculations (mentioned in the depth stage portion above), choose option 1.  If one bank is shorter than the other at the height you prescribe, the program automatically assumes the shorter bank to be vertical.  If you only want to analyze the cross section at data points available, select option 2.  This stops the at-a-stage calculations at the highest point where each riverbank has measured data.  This automatic stopping is a reasonable choice because it seems that water would rarely flow over the shorter bank, and therefore the power regression analysis should stop at the highest point of the shorter bank.  There are certainly exceptions to this, especially if you desire a more complete, full analysis of the cross section. If you would like to use all of the data available, select option 3.  This stops the at-a-stage calculations at the very highest point on the cross section.  If the riverbanks do not match, the shorter bank is assumed to be completely vertical.

After all of these choices are made and entered, the figure window will reappear, showing the stages where the power regression is being analyzed by placing a red dot at the depth increment until the specified height is reached from the previous menu.  Finally, the program will show the values of c and f from the power relation between Q and the hydraulic radius, R, where R = cQf and between Q and the hydraulic depth, H, where H = cQf.  The hydraulic depth values of c and f are typically similar and shown because they can be used later for the calculation of the nonuniform terms in Eq.1.

## Entering mean grain size (D50)

The mean grain size is called D50 signifying the 50th percentile of the diameters of the grain size data.  This value is used to calculate shear stress and nondimensional flowrate, Q*.

If you already know the value of D50 for the cross section under consideration, select option 1 at the Mean Grain Size menu.  Please note that the value you enter MUST be in millimeters as that is the standard measurement for this quantity.

Alternatively, the program can calculate the value of the mean grain size based on the number of grain size measurements that lie within standard ranges.  If you have raw data of grain size measurements, select option 2 under the mean grain size menu.  Next, the program will ask you how many measurements are less than 8 mm, how many measurements lie between 8 and 11.3 mm, how many measurements lie between 11.3 and 16, and so on.  After all of the data is entered, the program calculates a cumulative distribution to compute the mean grain size.  The value will be shown on the screen and the program will move on to the next step.

## Entering daily flow data

As mentioned previously, the daily flow data in the river is needed to compute the daily shear stress.  Therefore, you need to enter the river’s flow data as an input file.  Like the cross sectional geometry input data file for the stage-discharge power regression, the input file for the flow data must be properly formatted.  The Functional Flows program is highly sensitive to any deviations from the expected daily flow input format.

To input a daily flow data file, the file must have four (4) columns separated by spaces, tabs, or a combination of the two.  The first three columns correspond to the date as in month, day, and year (format:  mm/dd/yyyy).  The fourth column is the flowrate.  A quick reference is shown in figure 3:

Column 1	Column 2	Column 3	Column 4

Month		   Day		   Year		Flowrate

Figure 3:  Summary of daily flow data file format.

The starting date needs to be Julian Day 1, which is October 1st (or 10/01).  For the analysis of one (1) water year, the ending date must be September 30th (or 09/30) of the following year.  Therefore, you will have 365 entries if the year is not a leap year and 366 entries if the year is a leap year.  For the analysis of multiple water years, simply add as many years as desired immediately following September 30th of the first year.  Of course, these water years also start on October 1st and end of September 30th.  The program will automatically detect the number of years and account for leap years.  Therefore, it is extremely important that there be no gaps in this daily flow data.  Also, the years must be in chronological order and must be a four (4) digit year that increments on January 1st of each year.

The fourth and final column is the flowrate in the river.  Be very careful to ensure that the flowrate values are all in ft3/s (cubic feet per second, or cfs) if your unit system is English, or m3/s (cubic meters per second, or cms) if your unit system is Metric.

Figure 4 is an example of a properly formatted daily flow input file:

10	1	1963	449.81

10	2	1963	441.44

10	3	1963	434.22

10	4	1963	426.88

10	5	1963	422.56

10	6	1963	409.38

10	7	1963	394.72

…

9	24	1964	461.06

9	25	1964	464.72

9	26	1964	461.72

9	27	1964	450.81

9	28	1964	443.28

9	29	1964	446.19

9	30	1964	446.19

Figure 4:  An example of a properly formatted daily flow input file.

Note that leading zeros for any of the columns are not necessary, but also will not produce any errors while running the program.  Again, note that the file starts with 10/01 on a certain year and ends with 9/30 of the following year.  River flow data can typically be found at the USGS website(http://waterdata.usgs.gov/nwis/rt).

Once you have a properly formatted flow input data file, copy this file into the folder that contains the files for the Functional Flows program.  When prompted, enter the name of the file including the extension in the Matlab command prompt.  For example, if you wanted to use a data file named test_flow, you would enter test_flow.dat in the Matlab command prompt when asked for the name of the flow data file.  If you do not include the file extension or forgot to copy the file into the Functional Flows directory, an error will occur because the program cannot find and load that file.

## Entering extra shear stress terms

The Functional Flows program gives you the option of including some or all of the non-uniform terms from Eq. 1 in the Introduction section.  If non-uniform/non-steady terms are not added, the program uses by default the depth-slope product for the shear stress, shown in equation 3:

τ0=ρcQfSρs−ρD50	(3),

where the substitution R = cQf is made.

If you would like to use any of the non-uniform/non-steady terms select option 1 under the Extra Shear Stress Terms menu.

### The non-uniform change in height term, dh/dx

The change in height term is a measure of the change in height divided by the change of distance between an upstream cross section and the downstream cross section which is the site at which the analysis is being performed.  Therefore, to include the change in height term, you must have knowledge and data of this upstream cross section.  In order to include this non-steady term, you should either have the X, Y geometry of the pool or the c and f coefficients.

The first step after selecting to include the change in height term is the entry of the distance between the pool and the riffle.  Ensure that the value you enter is again in the proper units, either feet or meters.

The next step is the acquisition of the c and f coefficients for the upstream geometry.  The program uses the values of c and f based on H to replace the differential term dh/dx with Eq. 4:

dhdx=HR−HPdx=cRQfR−cPQfPdx	(4),

where:

HR is the hydraulic depth of the downstream cross section,

HP is the hydraulic depth of the upstream cross section,

cR and fR are the hydraulic geometry constants based on HR,

cP and fP are the hydraulic geometry constants based on HP, and

dx is the distance from the riffle to the pool.

If you already know the values of the hydraulic geometry constants based on H for the upstream cross section, select option 1 to manually enter these values.  Remember that these calculations need to be in the correct unit system.  If you only have the raw cross sectional data for the pool, ensure it is properly formatted by reading the “Entering hydraulic geometry power regression data” section above.  Then select option two to have the program calculate these values automatically.  Make sure that the input file has been copied into the folder that contains the Functional Flows files.  Next, the slope of the pool’s water surface is needed.  Refer to the “Entering water surface slope” section above to continue.  Now you must enter the name of the upstream cross sectional geometry file INCLUDING the file extension into the command prompt.  Review the “Entering hydraulic geometry power regression data” section to properly input the depth increment, the Manning’s n, and your choice of at-a-stage analysis termination.

If you have not entered X, Y geometry for the site under analysis, the program will next ask for the values of the hydraulic geometry constants c and f.  You have three options at this point.  Select option 1 if you would like to use the values of c and f based on the hydraulic radius that you entered before.  Typically, the values of c and f do not vary greatly when based on the hydraulic radius as compared to basis on the hydraulic depth.  For greater accuracy, you may enter different values of c and f based on the hydraulic depth of the riffle by selecting options 2 or 3.  If you select option 2, you need to manually input these values.  If you select option 3, you need to provide the cross sectional geometry of the riffle and the program will calculate the necessary values.

With all the necessary data collected, the program will output a message that it is calculating the change in depth term.  Upon completion (typically momentarily), the program will display a completion message and send you back to the extra shear stress terms menu.  If you do not wish to include any more terms, select option 4 and the program will progress.

### The non-uniform change in velocity term, U/g*dU/dx

The change in velocity term is measured again between the cross section being analyzed and an upstream cross section.  Unlike the change in depth term however, the cross sections of both sites are needed to complete the calculation of the change in velocity term.  Both geometries are required because the program uses the substitution in Eq. 5 for the velocity terms:

U=QA	(5),

where:

U is the velocity, and

A is the area of the cross section in question.

Because the daily flowrate Q has already been entered, the program only needs the area of the cross sections to arrive at the velocity.  Because the mean daily flow in the river changes each day, the submerged area of both cross sections will be different each day.  Also because the flowrate is different, the velocity will be different each day.  Therefore, the program needs the cross sectional geometry of the upstream and the downstream site to compute the change in velocity term between the riffle and the pool for each day the data exists.

If you do not have the cross sectional geometry of one of the cross sections, do not choose to include the change in velocity term under the extra shear stress terms menu.

If you do have geometries of both cross sections and have not chose to include the change in depth term discussed previously, the first thing the program will ask you after choosing option 2 is the distance between the cross sections.  Ensure your measurement is in the proper units system.  If you already have input data for the change in depth term, the program will store the value of dx and not prompt you for it again.

The change in velocity term requires the hydraulic geometry constants c and f based on the hydraulic radius of the pool.  If you have already entered the upstream cross sectional geometry for the purposes of the change in height term, you will not need to enter these values because the program will calculate and store these automatically.  If you have not already done this, you will need to either manually enter these values or enter the cross sectional geometry of the upstream cross section at this point.  If you wish to manually enter them, remember that they are now based once again on the hydraulic radius R, not the hydraulic depth H from the change in depth term.  To enter them manually, select option 1 when prompted for these values.  Again, ensure that the calculations to come by these numbers were in the correct units system.  However, since you need to enter the upstream cross sectional geometry anyway, it is recommended that you choose option 2 and have the program calculate the hydraulic geometry constants c and f and store the cross sectional geometry for computation of the area.

At this point, if you have not done so already, the program will ask for you to input the cross sectional geometry of both cross sections.  Before entering anything in the command prompt, ensure these files are properly formatted cross sectional geometry files by referring to the “Entering hydraulic geometry power regression data” section.  If they are not already present in the Functional Flows program files folder, copy them there.  Again, remember to include the extension of the input files when entering the names in the command prompt.

Finally, the program will show a message that it is computing the change in velocity term.  When it is finished, a calculation finished message will appear in the command window.

### The non-steady term, 1/g*dU/dt

The non-steady term is a measurement of the change in velocity divided by the change in time.  Because flowrate (Q) measurements are available each day, the change in time (dt) is one day.  As explained in the non-uniform change in velocity term above, the program needs the cross sectional geometry of the site under analysis to calculate the velocity each day.  Therefore, do not include the unsteady term if you do not have the cross sectional geometry data.

If you would like to include the unsteady term, select option 3 in the extra shear stress terms menu.  If you have not previously entered the riffle’s cross sectional geometry, the program will ask you to enter the cross sectional geometry at this point.  Please refer to the “Entering hydraulic geometry power regression data” if you are unsure about file formatting and entering it into the command prompt.

## Entering the Functionality Table

The functionality table includes the two major criteria in determining weather or not a given day is actually functional in the analysis, as discussed in the General Summary.  Therefore, selection of the functionality table is important because it can drastically affect the outputs of the Functional Flows program.

The functionality table includes the names of all of the ecological functions of interest of the target species.  The table gives the Julian Day range of each ecological function.  Also, it shows weather or not each ecological function is functional for all ranges of nondimensional shear stress.

The program will give you three choices for entering the functionality table under the Functionality Table menu.  The first two choices are default, preprogrammed choices.  The first choice of functionality table has only two ecological functions:  Bed Occupation and Bed Preparation.  These account for the times when there is interaction with the river bed (bed occupation) and the times when there is not interaction with the river bed (bed preparation).  Fig. 5 is the default compact table that option 1 selects:

Range                                Ecological Functions

Bed Occupation                    Bed Preparation

Julian Day                    1-180                           181-366

0.06 < T < 0.10                0                                 1

0.03 < T < 0.06                0                                 1

0.01 < T < 0.03                1                                 0

T < 0.01                0                                 0

Figure 5:  Compact default functionality table from option 1.

Note the nondimensional shear range appears on the left column.  The functionality for each ecological function for each shear range is shown by the 1’s and 0’s in the next two columns.  A “1” represents functional and a “0” represents non-functional.  The days of the year the function runs is shown in the Julian Day range row.

The second choice of functionality table expands the Bed Occupation stage into three specific parts:  Spawning, Incubation, and Emergence.  Within these bed occupation stages, there is sufficient overlap, allowing for the observation of their functionality.  The second choice is referred to as the full default table and is shown below:

Range                                 Ecological Functions

Spawning     Incubation    Emergence Bed Preparation

Julian Day              330- 90        330-180         60-180         181-330

0.06 < T < 0.10          0                  0                  0                  1

0.03 < T < 0.06          0                  0                  0                  1

0.01 < T < 0.03          1                  1                  1                  0

T < 0.01          1                  0                  1                  0

Figure 6:  Full default functionality table from option 2.

If you do not wish to use either the compact (option 1) or full (option 2) default functionality tables, you have the option of creating a functionality table of your own.  Before you select option 3 in the Functionality Table menu, it is recommended that you generate the table physically on paper, in a text file, etc.  There are many entries to make when generating a custom functionality table that could become confusing without referring to the table elsewhere.

You first must enter how many ranges you want for shear stress.  For example, the full default table in figure 6 contains four ranges of the shear stress, even though the last one is simply a “less than” condition.  The program expects that the last range is a “less than” range as shown in Fig. 5 and 6.  Therefore, make sure your functionality table includes this condition.

Next, the program will ask you how many ecological functions you wish to track.  You may create and enter as many as you like.  For example, there are two ecological functions (Bed Occupation and Bed Preparation) in Fig. 5 and four ecological functions (Spawning, Incubation, Emergence, and Bed Preparation) in Fig. 6.  For formatting purposes, the functionality table will only output a block of four ecological functions, their functionality, and range of Julian Days before continuing on the next line.  Nevertheless, the program can theoretically track as many ecological functions as you can enter.

The next set of questions fills in the ranges for the shear stress, the bottom portion of the leftmost columns in Figs. 5 and 6.  The program will first ask for the high range of the nondimensional shear stress.  This value is the upper limit on the first shear range.  This corresponds to 0.10 in Figs. 5 and 6.  Depending on how many ranges for shear you have, the program will ask you for the next threshold for the shear.  This number is the low value on the first shear range and the high value on the second shear range.  This value is 0.06 in Figs. 5 and 6.  This format progresses until the last range is reached.  Now the program will ask for the low threshold on the shear.  This is the smallest value of the nondimensional shear stress that you would like to track.  In the case of Figs. 5 and 6, this value is 0.01.  Due to this format, you will always enter the same number of values for the shear stress as the number of ranges you have for shear stress unless you entered one range.  If you entered one range, you will need to enter two values which signify the high and low thresholds for the range.

You will now be asked to fill in the names and Julian Day ranges for each of the ecological functions you have decided to use.  For formatting and presentation purposes, it is recommended that you limit the length of the name of each ecological function to about fifteen (15) characters, including spaces.  You can certainly exceed this suggestion, but errors or undesirable formatting may occur.  After you enter the name of each ecological function, the program asks for the starting Julian Day of the function.  Next, it asks for the ending Julian Day for the function.  The starting Julian Day does not necessarily need to come before the ending Julian Day.  For instance, Spawning (from Fig. 6) starts on Julian Day 330 and ends on Julian Day 90 of the next water year.

Once you have entered the ranges of the nondimensional shear stress and the names and Julian Day ranges for each ecological function, it is time to enter the actual functionality.  To avoid asking for each “1” or “0” separately, the program requires input of the functionality as a row of made up of only zeros and/or ones.  Matlab reads this group in as a vector and it is therefore REQUIRED that you place brackets around each functionality row.  The functionality rows must be separated by either as many spaces as you like or single commas (with or without spaces between commas and entries).  Fig. 7 shows examples of the only proper ways to enter functionality row data:

[0 1 1 0 1 0 0]	or	[0,1,1,0,1,0,0]

Figure 7:  Examples of proper row functionality data entry.  Note the brackets must be in place around the zeros and ones.

You will need to enter as many rows of functionality as there are ranges of nondimensional shear stress.  Remember that functionality is either a “1” or a “0” meaning functional or nonfunctional, respectively.  If you enter anything other than this, you will be asked to enter it again after a reminder message.  Also, the number of entries in each row must equal the number of ecological functions you have entered.  If you enter this incorrectly, you will be prompted to enter just that row again after a reminder message.  As a final example, to properly enter the functionality from Fig. 6, you would enter the rows shown in Fig. 8:

[0 0 0 1]	 	 [0, 0, 0, 1]

[0 0 0 1]	or	 [0, 0, 0, 1]

[1 1 1 0] 		 [1, 1, 1, 0]

[1 0 1 0]		 [1, 0, 1, 0]

Figure 8:  Proper entry of functionality rows from figure 6.

After this final step, the program will compile the table and display it in order that you can double check your data input.

## Visualizing the Inputs

After you have entered a functionality table, the program will ask if you would like to see any of your inputs either graphically or outputted to the screen.  Choose option 1 under the Visualizing the Inputs menu if you would like to see your inputs or choose option 2 if you wish to skip this step.  If you select option 1, a new menu appears with all of the available inputs to see.  This submenu will appear every time after you visualize an input until you select option 6 which exits this menu and proceeds with the calculations.

If you have previously entered cross sectional geometry of the site under analysis and would like to see the X, Y graphical representation of it, select option 1 on this new menu.  A figure window will appear showing the cross sectional geometry.  If you have not entered cross sectional geometry, a simple message will appear stating that there is no geometry to display.  The submenu will appear again until you select option 6 which signifies you do not wish to see any additional inputs.

Option 2 on this submenu is exactly like option 1 except now the upstream site’s cross sectional geometry will be displayed if you previously entered it.  Again, a simple message will state there is no geometry to display if you selected option 2 without entering upstream site information.

If you would like to see the hydrograph, the flowrate for each day in the water year, select option 3 on the submenu.  A figure window with the hydrograph will pop up and the submenu will reappear.

If you would like to see the functionality table you entered the step before, select option 4.  The functionality table will print to the command window in Matlab just as it did after the initial selection.

Option 5 is a graphical interpretation of the range of Julian Days for each ecological function present in the functionality table.  Each ecological function corresponds to a horizontal line along this graph.  The x-axis is the Julian Day for each water year.  The blue dots indicate the days where the ecological function occurs.  Fig. 9 below shows the graph of the ecological functions from the functionality table displayed in Fig. 6.

Figure 9:  Graph of the ecological functions from the full default functionality table of Fig. 6.

To ensure your data entries are correct, it is helpful to visualize your inputs and inspect them for any errors or inconsistencies.

## Changing Inputs

After you have been given a chance to see the inputs you have entered, the program asks if you would like to change any of these inputs.  If you are unsure about any inputs, scroll back up the command window and recall what values you entered for each input.  If something is awry, select option 1 under the Changing Inputs menu.  If everything is as you planned it, select option 2 to proceed to the calculations.

If you select option 1, the menu is very self-explanatory.  Choose from options 1 through 8 to change any previous inputs.  When you choose to change something, the exact same menus and formats will appear as in the previously described sections.  After you are finished changing everything you wish, select option 9 to proceed.

After you select option 9 in the submenu, you will be asked again if you would like to see any of the inputs visually.  If you would like to verify any of these changes, select option 1.  If not, select option 2 to proceed.

## Interpreting the outputs

After you have checked and/or changed your inputs, the program will display a message stating that it is calculating the inputs.  From here, a series of graphs will display in figure windows from Matlab.  Because some figures may contain lots of data and formatting, it is recommended that you maximize each figure window to fill the screen.  By doing so, you will have the best possible view of the data with enough space to see and read each phrase or indication on each figure.  There will be a minimum of seven different graphs that appear, some showing the same information in different ways.  As discussed above in the “Entering the Functionality Table” section, the ecological functions are broken down into groups of four if more than four ecological functions are present.  Because both of the default tables do not have more than four ecological functions, Matlab will output seven graphs. If you enter a custom functionality table that contains more than four ecological functions, Matlab will output more than seven graphs.

If any or all of these graphs display little or no information, it means there are no geomorphically or ecologically functional days for that specific analysis.

### The first set of figures

The first set of figures is titled “Geomorphically Functional Days” which means that the flow is sufficient to fall within a range of shear stress and shows a blue dot for every ecological function for every day in the entire analysis.  The very top graph on this set of figures is a graph of the nondimensional shear stress (shown in a blue line) at for every Julian Day in the analysis.  The shear is on the Y axis and the Julian Day lies on the X axis.  The constant black horizontal lines show the thresholds of the shear as indicated in the functionality table.  The gaps between these lines are the ranges of shear stress.

The remaining graphs each correspond to the geomorphic functionality of one ecological function.  As the legend for each graph indicates, a blue dot at “1” means the day is geomorphically functional (GF) for that ecological function.  Similarly, a blue dot at “0” means the day is not geomorphically functional (NGF) for that ecological function.  If there are more than four (4) ecological functions, this figure is continued on the next figure and indicated with a title of “Geomorphically Functional Days (Continued)”.  Fig. 10 shows an example of the first set of figures.

Figure 10:  Example of the first set of figures:  Geomorphically Functional Days.

### The second set of figures

The second set of figures shows the identical information as the first set.  The only change is that non-geomorphically functional days are not plotted as blue dots in the ecological functions graphs.  As the legend states, blue dots only indicate geomorphically functional days for each ecological function.  Again, if necessary, this set will continue on the next figure with the title being “Geomorphically Functional Days (Continued)”.  Fig. 11 shows an example of the second type of figures.

Figure 11:  Example of the second set of figures:  Geomorphically Functional Days.

### The third figure type

The third figure type is a bar graph of the total number of geomorphically functional days for each ecological function.  Because the bar graph is one graph and not a series of five smaller graphs as before, there is no need to continue the bar graph on additional figure windows.

The geomorphic functionality bar graph is quite simple to read.  Each ecological function is given a number on the left side of the graph.  The corresponding name of the function is printed just to the right of the number.  The number of total geomorphically functional days is shown in parenthesis next to the name.  The total number of geomorphically functional days for all ecological functions combined is displayed in a small text box to the right of the graph.  Fig. 12 shows an example of the geomorphically functional bar graph.

Figure 12:  Example of the geomorphically functional days bar graph.

### The fourth set of figures

The fourth set of figures shows the ecologically functional days which means that the flow is sufficient to fall within a range of shear stress and that the time at which it is happening concurs with the ecological functions. These figures are displayed in exactly the same format as the first set of figures.  Fig. 13 shows an example of the fourth set.

Figure 13:  Example of the fourth set of figures:   Ecologically Functional Days.

### The fifth set of figures

Predictably, the fifth set of figures shows the ecologically functionality in the exact same manner as the second set of figures.  Fig. 14 shows an example of this.

Figure 14:  Example of the fifth set of figures:  Ecologically Functional Days.

### The sixth figure type

The sixth figure type is the ecologically functional bar graph.  The formatting and layout is exactly the same as the geomorphically functional bar graph.  Fig. 15 shows an example.

Figure 15:  Example of the Ecologically Functional Bar Graph.

### The seventh set of figures

The seventh set of figures graphs the nondimensional shear stress of each ecologically functional day at the corresponding nondimensional flowrate in a logarithmic scale.  The shear is on the Y axis with the flowrate on the X axis.  Like in the first, second, fourth, and fifth set of figures, the constant black horizontal lines are the shear thresholds from the functionality table.  Each ecological function has a different color, and the legend will indicate what color represents what ecological function.  The legend may be blocking some information.  Simply click and drag to move the legend to a more appropriate space on the graph if needed.  Keep in mind that only ecologically functional days are plotted on this graph.  Therefore, a particular analysis may output many geomorphically functional days but still result in no ecologically functional days giving a blank final graphs.  An example of a graph with data is shown in Fig. 16.

Figure 16:  An example of the seventh type of figures:  Nondimensional shear versus nondimensional flowrate.

After all of the output figure windows are displayed, the program will output additional results to the command window in Matlab.  The first set of results show the actual functional flows for each range of shear stress.  These values come directly from the seventh set of figures.  The program tells you the lowest and highest nondimensional functional flow for each range of nondimensional shear stress.  If no flows are present in a given range, the program will say so.

The second group of data is the theoretical nondimensional functional flows result.  The program extrapolates the data out to determine what the value of the nondimensional flow would need to be in order to be ecologically functional for each specific range of shear stress.  When dealing with some of the non-uniform and non-steady terms, the shear stress may actually become negative.  When this happens, this extrapolation technique is invalid and the program gives a short message of this occurrence.

## The Output File

After all of the output graphs and the actual and theoretical functional flow ranges have been displayed, the program will ask you to input a name of a file to which all of the inputs and outputs are written.  Again, you must include a file extension, as the program reminds you.  Matlab will automatically create and write to this file which is stored in the Functional Flows folder.  If the file already exists, the old information will be erased and only the current data will be written.  For example, if you wanted the output file to be a Matlab output file named “test_run_1”, you would enter test_run_1.mat when asked for the output file name.

After you have entered the name of the file, you are given the option to name the specific analysis.  The name may be anything you wish containing as many different characters as you wish.  The name is a simple heading at the very top of the output file.  If you would like to enter a name for the analysis, select option 1.  If you select option 2 for no name, the output file will have a default title.  Fig. 17 shows the beginning of output files with and without names.

<<<< Output Data for First Test >>>>		A.

<<<< Output Data >>>>				B.

Figure 17:  Examples of headings for a named analysis (A.) and unnamed analysis (B.).

The layout and formatting of the output file was design for ease of readability.  There are no unnamed or unmarked columns of copious sets of data.  Each input is titled and labeled with units were applicable, and each output is clearly indicated.  Fig. 18 lists all the data, in order, you can expect to find in the output file.

Section 1:	Heading

Section 2:	Date and Time of file creation

Section 3:	Units

Section 4:	Water Surface Slope

(for upstream and downstream sites, if available)

Section 5:	Cross sectional geometry

(for upstream and downstream sites, if available)

Section 6:	Power regression data

(for upstream and downstream sites, if available)

Section 7:	Average grain size (D50)

Section 8:	Functionality Table

Section 9:	Scorecard with outputs for each day in analysis

Section 10:	Total days geomorphically and ecologically functional

Section 11:	Actual and Theoretical Functional Flows ranges

Figure 18:  Summary of the output file.

All of the longer columns of data in the output file have been formatted with tabs so that the data can be easily read by other programs such as Microsoft Excel.

## Sensitivity Study

Once the output file has been written, the Sensitivity Study menu will appear and asks if you would like to change one or more inputs for the purposes of a sensitivity study.  This can be very useful and informative because the study allows you to see how changing one input affects the output as a whole without going back and re-entering most of the same entries for a second analysis.  If you choose to run a sensitivity study, the program saves all of your previous inputs, only makes changes to the variables you indicate, and re-runs the entire analysis complete with a new output file.

If you would like to run a sensitivity study, choose option 1 under the Sensitivity Study menu.  The submenu shows you all of the available inputs to change.  Any selection you make here will take you back through the exact same steps as you have done before.  If you have any concerns working in this submenu, look to the previous sections for information.   The sensitivity study menu will prompt after every analysis, allowing you to change as many variables as you would like as many times as you would like.  Simply choose option 2 under the sensitivity study menu to finish the program.

# Tips and Troubleshooting

To exit the program at any time during its execution, hold the ‘control’ key and press ‘c’.  Any Matlab script or program can be terminated in this fashion.

If you have entered cross sectional geometry for the program to compute the values of c and f from the power regression and have entered a high stage and a low stage water surface slope, the nondimensionalized graph of the shear versus the flowrate may not follow a smooth trend.  To see a better trend, a smoother line, consider entering a smaller value for the power regression stage increment.  The interpolation of the water surface slope at each stage is based on the stage increment.  If the increment is smaller, there will be more entries and fewer gaps in the interpolation resulting in a smoother output graph.

When the program is calculating the outputs and graphing the results, a very rare error may occur, terminating the program.  Fig. 19 shows the error message.

???  In an assignment  A(I) = B, the number of elements in B and

I must be the same.

Error in ==> outputs at 655

[Xo(i),Yo(i)] = mminvinterp(q_test,yp,t_test(i));

Error in ==> FF at 268

[figure_counter,GMF,real_F,total_days_GMF,total_days_F,t_store,Q_store,tau_is_pos] ...

Figure 19:  Rare error during calculation of outputs.

If this happens, there is a simple fix needed in the actual Matlab code.  From the Matlab main window, click the open file button.  From here, all of the Functional Flows should appear because that is the directory in which you are currently working.  Select “FF” and click open.  Scroll down to line 240.  Try first by commenting out this line, that is, put a percent sign (%) in front of the line of code.  Save the file.  Run the program again.  If it happens again, uncomment the line of code (delete the percent sign) and change the number (default is 10) after “Qtest(j),” to a higher number, say 15 perhaps.  Save and run the file.  One of these changes should result in a success.

To ensure you see all of the data on each graph, maximize all the windows.  Also, move the legends around by clicking and dragging them to new locations to reveal data that may be hidden.

# Matlab Coding Acknowledgements

The function “mminvinterp” used in Functional Flows was downloaded and used with permission from the Matlab Central File Exchange website.  The algorithm is from the text “Mastering Matlab 7”.

The function “Roundoff” used in Functional Flows was downloaded and used with permission from the Matlab Central File Exchange website.  The function was written by Glenn Gomes.

The algorithm for reading in and properly displaying the functionality table was written by Chris Sullivan at UC Davis, 2007.